CHAPTER 6 : BOOLEAN ALGEBRA 3

Chapter Objectives

At the end of this chapter, you would have learnt:

- ‘QUINE-McCLUSKEY’ method of simplifying logic expressions;

- the use of ‘PROGRAMMABLE LOGIC ARRAYS’ (PLAs).

Topics

- Quine-McCluskey

- Programmable Logic Arrays

6.1	Quine - McCluskey Method

- K-map method becomes difficult as the number of variables increase.

- This method is based on arranging minterms according to number of 1’s in their binary representation, and eliminating variables by repeated combination of minterms. For e.g.: x1x2 + x1x2 = x1.

- Two minterms will combine as above only if they differ in one bit position in their binary representation.

- After every combination, the eliminated variables are represented by “-”. The combined minterm when no further combinations are possible, or, a minterm that did not combine with any other are called the ‘prime implicants’.

- Example: Consider :  m2, m3, m6, m7, m8, m9, m13, m15

Step 1:

| Minterm | Binary Representation | Combination I | Binary Rep. | Combination II | Binary Rep. |
| --- | --- | --- | --- | --- | --- |
| (One 1) m2  m8  | 0 0 1 0 1 0 0 0 | m2, m3  m2, m6  m8, m9 | 0 0 1 - 0 - 1 0 1 0 0 - | m2, m3, m6, m7 m2, m6, m3, m7 | 0 - 1 - 0 - 1 - |
| (Two 1’s) m3  m6  m9  | 0 0 1 1 0 1 1 0 1 0 0 1 | m3, m7  m6, m7  m9, m13 | 0 - 1 1 0 1 1 - 1 - 0 1 |  |  |
| (Three 1’s) m7  m13  | 0 1 1 1 1 1 0 1 | m7, m15 m13, m15 | - 1 1 1 1 1 - 1 |  |  |
| (Four 1’s) m15  | 1 1 1 1 |  |  |  |  |

Note:	-	Combined minterms are marked tick.

-	After combining the combined minterms are still grouped according to the number of 1’s.

-	‘Prime implicants’ are boxed.

Step 2:

- Cover all minterms using the minimum number of prime implicants, in the matrix shown below.

Minterms

‘Prime

Implicants’

m2	m3	m6	m7	m8	m9	m13	m15

|      | A (m2,m6,m3,m7) |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|    | B (m8,m9) |  |  |  |  |  |  |  |  |
|   | C (m9,m13) |  |  |  |  |  |  |  |  |
|   | D (m7,m15) |  |  |  |  |  |  |  |  |
|    | E (m13,m15) |  |  |  |  |  |  |  |  |

Step 3:

- Look at a column with a single cross mark. That prime implicant is ‘essential prime implicant’ since only that can cover the concerned minterm.

- Example:

A	is essential to cover m2, m3 and m6.

Also covers m7.

B  	is essential to cover m8.

Also covers m9.

- Tick mark the essential prime implicants (e.g.   A	and    B    ) and all the minterms covered by them.

- Only m13 and  m15 left now. They can be covered by    C     and     D    together, or, by    E    alone, chooses    E    ; tick    E     and m13 and m15.

- All minterms covered.

... Logic expression

F 	=	A	+	B	+ 	E

=	(m2, m3, m6, m7) + (m8, m9) +(m13, m15)

=	 x1 x3  +  x1 x2 x3   +  x1 x2 x4

6.2	Programmable Logic Arrays (PLAs)

- PLAs provide a simple way of implementing logic functions that are complex. For example, say, fourteen input variables and eight output functions.

- It is made possible because of availability of large-scale integrated circuit chips.

- A ‘PLA’ has a switching elements available at each cross point of “AND” & “OR” arrays. Until programmed cross point is not a conducting path.

6.2.1	Logic Gate Equivalent of a ‘PLA’

An Example of a ‘PLA’

6.3	Exercises

- Determine the prime implicants of the function

f (w,x,y,z) =   (1,4,6,7,8,9,10,11,15)

- Solve by Quine Mccluskey method and derive the logic expression for the five variables sum-of-product given below.

- The second part of summation denotes Don’t Care conditions.

F = (m4, m5, m10, m11, m15, m18, m20) + (Don’t Care)  (m9, m12, m14, m16)