CHAPTER 8 : COMBINATIONAL CIRCUIT 1

Chapter Objectives

At the end of this chapter you would have learnt:

- to design half adders, full adders, n-bit parallel adders and subtractors.

Topics

- Half Adder

- Full Adder

- Parallel Adder

- Parallel Subtractor

- Parallel Adder/Subtractor

- Overflow Detection

8.1	Half Adder

- Description:

- Two inputs: A, B.

- Two outputs: C(Carry), S(Sum).

- No carry-in.

- Block Diagram

- Truth Table

A	B	C	S

0	0	0	0

0	1	0	1

1	0	0	1

1	1	1	0

- Logic Diagram

C  = AB

S  = A  B

8.2	Full Adder

- Description:

- Three inputs: A, B, Ci(Carry-in).

- Two outputs: Co(Carry-out), S(Sum).

- Block Diagram

- Truth Table

A	B	Ci	Co	S

0	0	0	0	0

0	0	1	0	1

0	1	0	0	1

0	1	1	1	0

1	0	0	0	1

1	0	1	1	0

1	1	0	1	0

1	1	1	1	1

AB		AB

Ci	Ci

|  | 00 | 01 | 11 | 10 |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 0 |

Co = AB + ACi + BCi	S = A  B  Ci

- However, if we use boolean algebra to solve Co instead:

Co	= 	ABCi + ABCi + ABCi + ABCi

= 	Ci(AB + AB) + AB(Ci + Ci)

= 	Ci(A  B) +AB

- Logic Diagram

Example:

Implement FA(Full Adder) using HAs(Half Adders)

8.3	Parallel Adder

- Description:

- Two n bit inputs: X, Y.

- One n bit + carry bit output: S, Co.

- Adds two n bit inputs into one n bit output with carry.

- Addition at each stage depends on Ci from previous.

- 2’s complement inputs and output.

- Block Diagram (4 bit parallel adder)

- Logic Diagram (4 bit parallel adder)

Examples:

X + Y = 2 + 3 = 5

4 bit parallel adder: X = 00102 (+210), Y = 00112 (+310)

X + Y = 3 - 4 = -1

4 bit parallel adder: X = 00112 (+310), Y = 11002 (-410)

8.4	Parallel Subtractor

- Description:

- Two n bit inputs: X, Y.

- One n bit + carry bit output: S, C0.

- Subtract two n bit inputs into one n bit output with carry.

- 2’s complement inputs and output.

- Block Diagram (4 bit parallel subtractor)

- Logic Diagram

Examples:

X - Y = 4 - 3 = 1

4 bit parallel subtractor: X = 01002 (+410), Y = 00112 (+310)

X - Y = 4 - (-3) = 7

4 bit parallel subtractor: X = 01002 (+410), Y = 11012 (-310)

8.5	Parallel Adder/Subtractor

- Description:

- Two n bit + control inputs: X, Y, A/S.

- One n bit + carry bit outputs: S, C0.

- Performs addition when A/S is 0, subtraction when A/S is 1.

- 2’s complement inputs and output.

- Block Diagram (4 bit parallel adder/subtractor)

- Design Issue

Y	A/S	    Y’

0	  0	    0

0	  1	    1

1	  0	    1

1	  1	    0

- Logic Diagram (4 bit parallel adder/subtractor)

Examples:

X + Y = 5 + 2 = 7

4 bit parallel adder/subtractor: X = 01012 (+510), Y = 00102 (+210), A/S = 0.

X - Y = 5 - 2 = 3

4 bit parallel adder/subtractor: X = 01012 (+510), Y = 00102 (+210), A/S = 1.

8.6	Overflow Detection

- Description:

- Three sign bit inputs: SA, SB, SC.

- One control output: O.

- Overflow is NOT Carry.

- Detects overflow whenever the sign of sum is different from the like signs of the two numbers.

- Only two conditions:

1. 	SA	=	SB	=	0	and	SC	=	1

2. 	SA	=	SB	=	1	and	SC	=	0

Therefore,

O = SASBSC + SASBSC.

- Block Diagram

- Logic Diagram

8.7	 Exercises

- Design a 3 bit adder/subtractor with inputs A2, A1, A0, B2, B1, B0 and A/S; outputs S2, S1, S0 and Cout.  The adder/subtractor should perform:

Addition of A2, A1, A0 with B2, B1, B0 when A/S is 1, and

Substraction of A2, A1, A0 with B2, B1, B0 when A/S is 0.

- Design a 4 bit parallel adder to perform the following operations.

- double the inputs A3A2A1A0.

- increase the inputs by 1 if B is an odd number otherwise the output does not change.

(Assume the inputs are A3A2A1A0 and B3B2B1B0)