CHAPTER 12 : SEQUENTIAL CIRCUIT 3

Chapter Objectives

At the end of this chapter, you would have learnt:

- what ring counter, synchronous binary counter, asynchronous binary counter are and their applications.

Topics

- Ring counter

- Synchronous binary counter

- Asynchronous binary counter

12.1	Ring Counter

- Description:

- Counter must be pre-set such that one of the bit is unique,

- i.e. 00012 or 11102.

- Used whenever only one output is desirable at any single time.

- Can also be used to “step-down” the frequency of pulses.

Logic Diagram (4-bit ring counter)

- Application

- Traffic lights

- Assume that Red, Green and Amber last for 20 seconds each, hence, CLK shall be 120 Hz.

- For Red and Green to last for 20 seconds each; Amber for 2 seconds, then CLK supplier becomes 12 Hz. At the same time, use 10 flip-flops each for Red and Green, and Amber remains as 1 flip-flop.

- Outputs of 10 Red OR-ed; outputs of 10 Green OR-ed; Amber AND-ed with 12 Hz Clock for it to flash.

12.2	Synchronous Binary Counter

- Outputs follow a defined counting sequence.

- Clock signal initiates all flip-flops to change state together.

- Also known as parallel counter.

Example 1:

Using T flip-flop to implement 4 bit synchronous binary-up counter.

Logic Diagram (4-bit parallel counter)

- The Q outputs of the flip-flops count from 00002, 00012, ..., 11112, and back to 00002.

- To count down 11112, 11102, ..., 00002; use Q instead.

- 4 bit synchronous binary-up counter.

Clock Pulse		B3	B2	B1	B0

0		 0	 0	 0	 0

1		 0	 0	 0	 1

2		 0	 0	 1	 0

3		 0	 0	 1	 1

4		 0	 1	 0	 0

5		 0	 1	 0	 1

6		 0	 1	 1	 0

7		 0	 1	 1	 1

8		 1	 0	 0	 0

9		 1	 0	 0	 1

10		 1	 0	 1	 0

11		 1	 0	 1	 1

12		 1	 1	 0	 0

13		 1	 1	 0	 1

14		 1	 1	 1	 0

15		 1	 1	 1	 1

B0 toggles on every

clock pulse

B1 toggles whenever B0 = 1

no change when B0 = 0

B2 toggles whenever B0 B1 =  1

B2 toggles whenever B0 B1 =  1

Example 2:

Using JK to implement 6 bit synchronous binary-up counter.

- 6 bit synchronous binary-up counter

- To implement binary-down counter;

- Complement the output sequence

- (i.e. change all 0’s to 1’s and all 1’s to 0’s)

- Take outputs from Q instead of Q

| Clock Pulse | B5 | B4 | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 0 | 0 | 1 | 1 |
| 4 | 0 | 0 | 0 | 1 | 0 | 0 |
| 5 | 0 | 0 | 0 | 1 | 0 | 1 |
| 6 | 0 | 0 | 0 | 1 | 1 | 0 |
| 7 | 0 | 0 | 0 | 1 | 1 | 1 |
| 8 | 0 | 0 | 1 | 0 | 0 | 0 |
| 9 | 0 | 0 | 1 | 0 | 0 | 1 |
| 10 | 0 | 0 | 1 | 0 | 1 | 0 |
| 11 | 0 | 0 | 1 | 0 | 1 | 1 |
| 12 | 0 | 0 | 1 | 1 | 0 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 | 1 |
| 14 | 0 | 0 | 1 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 1 | 1 |
| 16 | 0 | 1 | 0 | 0 | 0 | 0 |
| . | . | . | . | . | . | . |
| . | . | . | . | . | . | . |
| . | . | . | . | . | . | . |
| 31 | 0 | 1 | 1 | 1 | 1 | 1 |
| 32 | 1 | 0 | 0 | 0 | 0 | 0 |

B0 toggles on every clock pulse

B1 toggles whenever B0 = 1 no change when B0 = 0

B2 toggles whenever B0.B1 =  1

B2 toggles whenever B0.B1.B2 = 1

12.3	Asynchronous Binary Counter

- Description:

- There are no clock signals to synchronise changes.

- Outputs follow a defined counting sequence.

- Flip-flops do not change state together in that they have a different triggering.

- Also known as ripple counter.

Example:

Logic Diagram (4-bit ripple counter)

- Using T flip-flop to implement 4 bit ripple counter.

- The output of one flip-flop is connected as a clock to the next more significant flip-flop.

- The Q outputs of the flip-flops count from 00002, 00012, ..., 11112, and back to 00002.

- To count down 11112, 11102, ..., 00002; use Q instead.

Example:

- Using JK flip-flop to implement 4 bit ripple counter.

- Ripple (asynchronous binary) counter.

| Clock Pulse | B3 | B2 | B1 | B0 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 1 |
| 2 | 0 | 0 | 1 | 0 |
| 3 | 0 | 0 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 |
| 5 | 0 | 1 | 0 | 1 |
| 6 | 0 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 |
| 8 | 1 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 |
| . | . | . | . | . |
| . | . | . | . | . |
| . | . | . | . | . |
| 15 | 1 | 1 | 1 | 1 |

12.4	Exercises

- Draw a synchronous mod-8 binary counter using JK flip-flop.

- Draw an asynchronous mod-8 binary counter using JK flip-flop.