CHAPTER 3: FLOATING POINT ARITHMETIC

Chapter Objectives

At the end of this chapter, you will learn to:

- represent numbers in floating point format;

- do different types of shifting;

- do floating point arithmetic.

Topics

- Floating Point Representation

- Logical Shifts

- Arithmetic Shifts

- Rotate Shifts

- Floating Point Arithmetic

3.1	Floating Point Representation

- Any binary number, N may be represented as

N	=	M x 2E

where	M		Mantissa

E		Exponent

- A floating point number needs to be normalised

- to avoid multiple representations of the same number;

- to maintain as many significant bits as possible in the mantissa.

- Condition for normalisation

0.12    |M| < 1.02

Example:	1001 11112 = 0.1001 1111 x 28

3.1.1	General Floating Point Format

| Mantissa | Exponent |
| --- | --- |

- Size and data format used for mantissa and exponent is system dependent.

Examples:

| 10 bit Mantissa 2’s Complement | 6 bit Exponent 2’s Complement |
| --- | --- |

| 10 bit Mantissa Sign-Modulus | 6 bit Exponent 2’s Complement |
| --- | --- |

| 12 bit Mantissa 2’s Complement | 4 bit Exponent Sign-Modulus |
| --- | --- |

Example:

Assume a 10 bit Mantissa and a 6 bit Exponent in 2’s complement, represent in floating point format the following numbers:

- 51010

- 0.251010

3.1.2	Floating Point

- Mantissa size affects precision

- Less bits => Less precision

- Exponent size affects range of magnitudes

- More bits => Bigger range

- Assume a 10 bit Mantissa and a 6 bit Exponent in 2’s Complement

- Maximum Positive Number

| 0 | 111111111 | 0 | 11111 |
| --- | --- | --- | --- |

= 	0.1111111112 x 231

= 	[1 - 2-9] x 231

= 	0.998 x 2,147,483,648

= 	2,143,188,680.70

| 0 | 100000000 | 1 | 00000 |
| --- | --- | --- | --- |

= 	0.12 x 2-32

= 	0.5 x 2-32

= 	0.5 x 0.0000000002

= 	0.0000000001

- Maximum Negative Number

| 1 | 000000001 | 0 | 11111 |
| --- | --- | --- | --- |

= 	1.0000000012 x 231

= 	-0.1111111112 x 231

= 	-(1 - 2-9) x 231

= 	-0.998 x 2,147,483,648

= 	-2,143,188,680.70

| 1 | 100000000 | 1 | 00000 |
| --- | --- | --- | --- |

=	1.1000000002 x 2-32

=	-0.12 x 2-32

=	-0.5 x 2-32

=	-0.5 x 0.0000000002

=	-0.0000000001

3.1.3	Summary

|		|	|		|

-2.143,188,680.70	-0.0000000001	0	0.0000000001	-2.143,188,680.70

3.2	Logical Shifts

- Logical Shifts multiply and divide unsigned binary integer numbers.

3.2.1	Logical Shift Left (LSL)

- For unsigned binary integer, if the bit shifted out is a ‘0’, the number is multiplied by two.

Example:	000110112	= 	27

After 1st LSL, it becomes

001101102	= 	27 x 2 or 54

After 2nd LSL, it becomes

011011002	= 	27 x 4 or 108

After 3rd LSL, it becomes

110110002	= 	27 x 8 or 216

Another additional LSL, however gives

101100002	= 	176

which is not equal to 27 x 16 because a ‘1’ bit was shifted out.

3.2.2	Logical Shift Right (LSR)

Example:	000110112	= 	27

Perform LSR three times results in

000000112	= 	3

which is the integer quotient of

2723 	= 	278	= 	3.38

3.3	Arithmetic Shifts

- Arithmetic Shifts multiply and divide 2’s complement numbers.

3.3.1	Arithmetic Shift Left

Example:	111001012	= 	-27

After 1st ASL, it becomes

110010102	= 	-27 x 2 	= 	-54

After 2nd ASL, it becomes

100101002	= 	-27 x 4 	= 	-108

Another additional ASL, however gives

101010002	= 	-88

which is not equal to -27 x 8

Explanation

Overflow occurs for negative numbers when a ‘0’ bit is shifted out of the position next to the sign bit.

Conclusion

- If the sign bit is zero (positive), when a ‘1’ bit is shifted out of the position next to the sign bit, then overflow will occur.

- If the sign bit is one (negative), when a ‘0’ bit is shifted out of the position next to the sign bit, then overflow will occur.

3.3.2	Arithmetic Shift Right

Example:	11100101	= 	-27

Perform ASR three times results in

11111100	= 	-4

which is the next smaller integer of

-2723	=	278	=	-3.38

3.3.3	Rotate Shifts

This operation may be used to inspect the content of a certain bit in the register.

Rotate Shift Right

Rotate Shift Left

3.4	Floating Point Arithmetic

- Addition of 2 Numbers.

Steps:

- Convert each number to their binary equivalent.

- Normalise the numbers.

- Represent the numbers in the required floating point format.

- For negative numbers, perform a 2’s complement on the mantissa.

- Compare the exponents and rescale the smaller one.

- Normalise the results.

Example:

By using floating point arithmetic with 10bit mantissa and 6 bit exponent format, both in 2’s complement form, show the result of the addition of 42.40436 and -9.87612. Convert the result to the decimal system.

Answer

42.40436	= 	4*61 + 2*60 + 4*6-1 + 0*6-2 + 4*6-3 + 3*6-4

= 	26.687510 	(Convert base 6 to decimal)	= 	11010.10112 	(Convert decimal to binary)	= 	0.1101010112*25 	(Normalised)

= 	0110 1010 11 000101 	(Put into 10-6 format)

-9.87612	= 	-(9*120 + 8*12-1 + 7*12-2 + 6*12-3)

= 	-9.7187510	(Convert base 12 to decimal)

= 	-(1001.10111)2 	(Convert decimal to binary)

= 	-(0.1001101112*24) 	(Normalised)

= 	-(0100 1101 11 000100)	(Put into 10-6 format)

= 	1011 0010 01 000100 	(2’s complement mantissa)

Rescaling the smaller exponent,

-9.87612	= 	1101 1001 00 000101	(Arithmetic shift mantissa right)

Adding the mantissa together,

0110 1010 11

+	1101 1001 00

- 0100 0011 11

Result	= 	0100 0011 11 000101	(Result is already normalised)

= 	0.1000011112*25	(Convert 10-6 format to floating point)	= 	10000.11112 	(Convert floating point to binary)

= 	16.937510 	(Convert binary to decimal)

3.5	Exercises

- What is the maximum and minimum number for both positive and negative range if the following floating point formats (both in 2’s complement) are used.

- 12 bit mantissa and 4 bit exponent.

- 11 bit mantissa and 5 bit exponent.

- The following binary numbers are in floating point representation (11-bit mantissa and 5 bit exponent) in 2’s complement.

- 0110 1110 0011 1010

- 1011 0010 0100 1001

- 1000 0101 0010 0000

What decimal numbers do they represent?

- Using the floating point format,

M (10 bit) 		E (6 bit)	 [Both in 2’s complement]

perform floating point arithmetic for the following decimal numbers.

- 21 + 5

- 20 – 4

- a.	By assuming the use of a 16 bit word with 10 bit for the (normalised) and 6 bit for the exponent and both 2’s complement format. Perform the addition of 2E.0A16 and -306.278 using floating point arithmetic. Show each step of your calculation.

- Using the floating point format M (12 bit) E (8 bit) both in 2’s complement.

- Determine the valid range of positive and negative numbers for the format you have defined. (State any assumptions made)

- Using the same 20 bit word, if you increase the number of bits used to represent the mantissa, what effect will this have on the floating point number you can represent?

- If you can increase the number of bits used to represent the exponent instead, what happens then?