CHAPTER 11 : SEQUENTIAL CIRCUIT 2

Chapter Objectives

At the end of this chapter, you would have learnt:

- Flip-Flop Conversions:

- SR to D, JK to T

- T to D, D to T

- SR to JK

- Shifting:

- Logical Shift Left and Right

- Rotate Shift Left and Right

- Arithmetic Shift Left and Right

Topics

- Flip-Flop Conversion

- Shifting

11.1	Flip-Flop Conversions

11.1.1	SR to D

S	R		Qt+1 		S	R	Qt+1		D	Qt+1

0	0	  	Qt 		0	1	   0		0	  0

0	1	  	0 		1	0	   1		1	  1

1	0	  	1

1	1	  	-

    - JK to T

J	K	 Qt+1 		J	K	Qt+1		T	Qt+1

0	0	  Qt 		0	0	  Qt		0	  Qt

0	1	  0 		1	1	  Qt		1	  Qt

1	0	  1

1	1	  Qt

11.1.3	T to D

Qt	D	XOR	Qt+1

0	0	  0	   0

0	1	  1 	   1

1	0	  1 	   0

1	1	  0	   1

11.1.4	D to T

Qt	T	XOR	Qt+1

0	0	  0	   0

0	1	  1 	   1

1	0	  1 	   1

1	1	  0	   0

11.1.5	SR to JK

S	R	Qt+1 			J	K	Qt+1

0	0	  Qt 			0	0	   Qt

0	1	  0 			0	1	   0

1	0	  1 			1	0	   1

1	1	  -			1	1	   Qt

11.2	Shifting

    - Logical Shift Left (LSL)

- Binary 0 is always introduced into the LSB(Least Significant Bit).

- MSB (Most Significant Bit) bit will be lost after shift.

- For a n-bit register, n or more times of LSL will result in all 0’s.

11.2.2	Logical Shift Right (LSR)

- Binary 0 is always introduced into the MSB.

- LSB bit will be lost after shift.

- For a n-bit register, n or more times of LSR will result in all 0’s.

11.2.3	Rotate Shift Left (RSL)

- MSB bit is introduced into the LSB.

- No bit is lost after shift.

- For n-bit register, multiple of n times of RSL will have no effect on the register.

11.2.4	Rotate Shift Right (RSR)

- LSB bit is introduced into the MSB.

- No bit is lost after shift.

- For a n-bit register, multiple of n times of RSR will have no effect on the register.

11.2.5	Arithmetic Shift Left (ASL)

- MSB bit (sign bit) is introduced into the LSB, without change of MSB.

- The bit just before MSB is lost after shift.

11.2.6	Arithmetic Shift Right (ASR)

- MSB bit (sign bit) is introduced into the bit after MSB, without change of MSB.

- The LSB is lost after shift.

11.3	Exercises

- Implement the D flip-flop using JK flip-flop.

- Implement the T flip-flop using SR flip-flop.

- Draw 4-bit LSL and LSR registers using SR flip-flops.

- Draw 4-bit RSL and RSR registers using T flip-flops.

- Draw 3-bit ASL and ASR registers using JK flip-flops.