CHAPTER 4 : BOOLEAN ALGEBRA 1

Chapter Objectives

At the end of this chapter, you will learn:

- basic logic functions, i.e., “logic gates” that serve as building blocks to realise more complex logic functions;

- the “truth table” representation of a logic function;

- the universality of “NAND” and “NOR” gates to realise any other basic logic functions;

- to see a few practical uses of basic logic gates;

- canonical methods of deriving a logic function from the truth table.

Topics

- Definition

- Truth Table

- Boolean Operators

- Applications

- Sum-of-Product Expression

- Product-of-Sum Expression

4.1	Definition

- “An algebra of logic used in the design of digital logic circuitry”.

- The logic circuitry implements a ‘boolean function’ defined as follows:

- a set of ‘boolean variables’ is taken as independent variables with possible values of ‘0’ or ‘1’.

- another boolean variable is assigned the role of a dependent variable.

- a rule is formulated which assigns a value to the independent variable for each set of values of independent variables.

4.2	Truth Table

4.2.1	Basic Structure of a Truth Table

| Inputs | Outputs |
| --- | --- |
| All possible combinations |  |

- For a 4-input variables Truth Table

A	B	C	D	Output Function, F

0	0	 0	0

0	0	0	1

0	0	1	0

0	0	1	1

0	1	0	0

0	1	0	1

0	1	1	0

24 = 16		0	1	1	1

combinations	1	0	0	0

1	0	0	1

1	0	1	0

1	0	1	1

1	1	0	0

1	1	0	1

1	1	1	0

1	1	1	1

4.2.2	Uses of Truth Tables

- Based on a given boolean expression

- Obtain the output for different inputs

- Given desired outputs

- Determine the expression needed in order to obtain the                                 outputs

4.3	Boolean Operators

4.3.1	“And” Operator

- The “And” operator:

- Operates on 2 or more variables

- An output of 1 when all inputs are 1’s

- Represented by “.”

- “AND” operation truth table:

| A | B | C = A . B |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 0 0 0 1 |

- Circuit Symbol:

- “AND” gate can have 2 or more inputs

4.3.2	“OR” Operator

- The “OR” operator:

- Operates on 2 or more variables

- An output of 1 when any or all inputs are 1’s

- Represented by “+”

- “OR” operation truth table:

| A | B | C = A + B |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 0 1 1 1 |

- Circuit Symbol:

- “OR” gate can take 2 or more inputs

4.3.3	“NOT” Operator

- The “NOT” operator:

- Operates on a single variable

- Produces opposite binary state of input

- Represented by “- ” (e.g. A ) or “‘” (e.g. A’)

- “NOT” operation truth table:

| A | A |
| --- | --- |
| 0 1 | 1 0 |

- Circuit Symbol:

- “NOT” gate can have only 1 input

4.3.4	“NAND” Operator

- The “NAND” operator:

- Operates on 2 or more variables

- Outputs 0 when all inputs are 1’s

- Performs AND then NOT function

- “NAND” operation truth table:

| A | B | C = A.B |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 1 1 1 0 |

- Circuit Symbol:

- “NAND” gate can have 2 or more inputs

4.3.5	“NOR” Operator

- The “NOR” operator:

- Operates on 2 or more variables

- Outputs 0 when any or all inputs are 1’s

- Performs OR then NOT function

- “NOR” operation truth table:

| A | B | C = A + B |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 1 0 0 0 |

- Circuit Symbol:

- “NOR” gate can have 2 or more inputs

4.3.6	“XOR” / “EX-OR” Operators

- The “XOR” / “EX-OR” operator:

- Operates on 2 variables

- Outputs 1 when inputs are not equal

- (i.e. it is a non-equivalence gate)

- Represented by “ ”

- “XOR” operation truth table:

| A | B | C = A B = AB + AB |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 0 1 1 0 |

- Circuit Symbol:

- “XOR” gate can have only 2 inputs

4.3.7	“XNOR” / “EX-NOR” Operator

- The “XNOR” / “EX-NOR” operator:

- Has 2 input variables

- Outputs ‘1’ when inputs equal (i.e. it is an equivalence gate)

- Represented by A  B or A  B

- “XNOR” operation truth table:

| A | B | C = A - B = A B = AB + A B |
| --- | --- | --- |
| 0 0 1 1 | 0 1 0 1 | 1 0 0 1 |

- Circuit Symbol:

- “XNOR” gate can have only 2 inputs

4.3.8	Nand Gate as Invertor

- All inputs are taken from the same variable

- (i.e. tie all the input lines to the same variable input)

- Output is inverted

4.3.9	NOR Gate as Invertor

- All inputs are taken from the same variable

- (i.e.: tie all the input lines to the same variable input)

- Output is inverted

- Conclusion

4.3.10	De Morgan’s Theorem

- Steps

- Invert all input variables

- Change “AND” operators to “OR” operators and “OR” operators to “AND”operators

Example:

F	=	A + BC + B CD + AB

=	A (B + C) (B + C + D) (A + B)

4.3.11	Dual Functions of a Nand Gate

- From De Morgan’s Theorem, we know A + B = A.B

-  given	F	=	A + B

= 	A + B

=	A.B (from De Morgan’s Theorem)

- Occurrence of A + B can be replaced by: A.B

- NAND gate is performing OR function

- 3 NAND gates needed to realise the OR function

- Circuit Symbols:

- Truth Table:

| A | B | A | B | F = A . B |  |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 | OR function A + B |
| 0 | 1 | 1 | 0 | 1 |  |
| 1 | 0 | 0 | 1 | 1 |  |
| 1 | 1 | 0 | 0 | 1 |  |

4.3.12	Dual Functions of a NOR Gate

- From De Morgan’s Theorem we know A.B = A + B:

Given   F	=	 AB

=	 AB

=	 A + B (from De Morgan’s Theorem)

- Occurrence of AB can be replaced by: A + B

- NOR gate is performing AND function

- 3 NOR gates needed to realise the AND function

- Circuit Symbols:

- Truth Table:

| A | B | A | B | F = A+ B | AND function A.B |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |  |
| 0 | 1 | 1 | 0 | 0 |  |
| 1 | 0 | 0 | 1 | 0 |  |
| 1 | 1 | 0 | 0 | 1 |  |

4.3.13	NAND Gate - UNIVERSAL Gate

- AND Function

- OR Function

- Invertor Function

4.3.14	NOR Gate - UNIVERSAL Gate

- AND Function

- OR Function

- Invertor Function

4.3.15	AND Operation on Variables

- Gives a product term

Example:

- Minterm

- A product term that contains all the variables or their inverse.

Example:

A B C D is a minterm (it is also a product term)

A C D is not a minterm (it is a product term)

4.3.16	OR Operation On Variables

- Gives a sum term

Example:

- Maxterm

- A sum term that contains all the variables or their inverses

Example:

(A + B + C + D) is a maxterm (it is also a sum term)

{A + C + D) is not a maxterm (it is a sum term)

4.4	Some Application

4.4.1	Decimal to Binary Encoder

- The desired decimal code is selected by closing the switch. The corresponding binary pattern appears as output y3 y2 y1 y0

4.4.2	AND Gate to Block / Transmit Data

- The “ENABLE” controls the contents of the register from appearing on output y3 y2 y1 y0.

4.4.3	Binary to Decimal or Instruction Decoder

- Depending upon the binary content of the register one of the AND gate output is selected.

4.4.4	Odd Parity Tester / Generator

- XOR output = 1 when register contains odd number of 1’s

4.4.5	Controller Inverter

- When ‘INVERT’ is high, the register contents are complemented and appear at y3 y2 y1 y0.

4.5	Sum-of-Product Expression

- Function is expressed as the canonical sum of minterms.

Example:

f(A, B, C)	= 	(0, 1, 3, 7)

=	A B C + A B C + A B C + ABC

Each product term must be a minterm.

Example:

f (A, B, C)	=	A B + A C + A B C

Note: 	Function F is not a canonical sum-of-minterms form because at least one of the product terms is not a minterm.

Example:

| Minterm Number | Product Term | A | B | C | Output f (A, B, C) |
| --- | --- | --- | --- | --- | --- |
| m0 m1 m2 m3 m4 m5 m6 m7 | ## ABC ABC ABC ABC | 0 0 0 0 1 1 1 1 | 0 0 1 1 0 0 1 1 | 0 1 0 1 0 1 0 1 | 1 1 0 1 0 0 0 1 |

- Sum-of-Minterms form

f(A, B, C)	=	 m0 + m1 + m3 +m7

= 	A B C + A B C + A B C + A B C

f(A, B, C)	= 	(0, 1, 3, 7)

- m = minterm

- Two level AND-OR implementation.

- Each term, e.g. A B C is a product hence the name SUM-OF-PRODUCTs.

- Sum-of-minterms form is also Sum-Of-Products form; but Sum-Of-Products form does not imply it is sum-of-minterms form.

4.6	Product-of-Sum Expression

- Function is expressed as the canonical product of maxterms.

Example:

f(A, B, C) 	=	(1, 5, 6)

=	(A + B + C)(A + B + C) (A + B + C)

Each sum term must be a maxterm.

Example:

f(A, B, C)	=	(A + B) (A + C) (A + B + C)

Note: 	Function F is not a canonical product-of-maxterms form because at least one of the sum term is not a maxterm.

Example:

| Maxterm Number | Sum Term | A | B | C | Output f(A, B, C) |
| --- | --- | --- | --- | --- | --- |
| M0 M1 M2 M3 M4 M5 M6 M7 | A + B + C A + B + C A + B + C A+B+C | 0 0 0 0 1 1 1 1 | 0 0 1 1 0 0 1 1 | 0 1 0 1 0 1 0 1 | 0 0 1 0 1 1 1 0 |

- Product-Of-Maxterms form

f(A, B, C)	= 	M0M1M3M7

=	(A + B + C) (A + B + C) (A + B + C) (A + B + C)

f(A, B, C)	=	(0, 1, 3, 7)

- M = maxterm

- Two Level OR-AND implementation.

- Each term, e.g. (A + B + C) is a sum hence the name PRODUCT-OF-SUMs.

- Product-of-maxterms form is also Product-of-Sums form; but Product-of-Sums form does not imply it is Product-of-Maxterms form.

4.6.1	SOP Expression from Truth Table

- Sum all the product terms where the output = 1.

Given the truth table:

| A | B | C | Output (F) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

- Write out the SOP form for F.

4.6.2	POS Expression from Truth Table

- Obtain the product for terms where the output = 0.

Given the truth table:

| A | B | ## C | Output (F) |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 1 |
| 1 | 0 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

- Write out the POS form for F.

4.7	Exercises

- Draw the truth table and write the “SOP” and “POS” form and logic function that will represent the truth table. Also draw the implementation circuits using “AND-OR” and “NAND-NAND” for “SOP” and using “OR-AND” and “NOR-NOR” for “POS”. The expressions are given as summation of minterms, for a variable functions.

- m0, m1, m3, m7

- m0, m1, m2, m4, m5

- m3, m5, m6, m7

- Obtain the truth table of the following functions and express each function in sum-of-minterms and product-of-maxterms.

- (xy + z) (y + xz)

- (A’ + B) (B’ + C)