CHAPTER 10 : SEQUENTIAL CIRCUIT 1

Chapter Objectives

At the end of this chapter you would have learnt :

- the idea of Flip-Flops and how to design SR, D, T and JK Flip-Flops

Topics

- Clock Circuitry

- Basic Flip-Flop Latches

- SR Flip-Flop

- D Flip-Flop

- T Flip-Flop

- JK Flip-Flop

10.1	Clock Circuitry

- Description:

- Pulses used to trigger the devices to accept input, typically used in synchronous system.

- Various type of triggering methods.

10.2	Basic Flip-Flop Latches

- Description:

- Able to maintain the signal by means of feedback from the output.

- Output depends on the previous state of the output.

- The simplest form of a memory cell.

- NOR Implementation

- Active-high inputs

Suppose that initially, Q is at high voltage (1) and Q is at low voltage (0).

At time = 0

- Suppose that both S and R at low voltage(0)

For NOR gate 1, 0 and 1 will give an output 0 (Q)

For NOR gate 2, 0 and 0 will give an output 1 (Q)

Input		     Output

Q	S	R		Q

1	0	0		1

At time = 1

- Suppose that S at low voltage(0) and R at high voltage(1)

For NOR gate 1, 0 and 1 will give an output 0 (Q)

For NOR gate 2, 0 and 1 will give an output 0 (Q)

then

For NOR gate 1, 0 and 0 will give an output 1 (Q)

For NOR gate 2, 1 and 1 will give an output 0 (Q)

Input		     Output

Q	S	R		Q

1	0	0		1

At time = 2

- Suppose that both S and R at low voltage(0)

For NOR gate 1, 0 and 0 will give an output 1 (Q)

For NOR gate 2, 1 and 0 will give an output 0 (Q)

Input		     Output

Q	S	R		Q

0	0	0		0

At time = 3

- Suppose that S at high voltage(1) and R at low voltage(0)

For NOR gate 1, 1 and 0 will give an output 0 (Q)

For NOR gate 2, 0 and 0 will give an output 1 (Q)

Input		     Output

Q	S	R		Q

0	1	0		1

At time = 4

- Suppose that S at low voltage(0) and R at low voltage(0)

For NOR gate 1, 0 and 1 will give an output 0 (Q)

For NOR gate 2, 0 and 0 will give an output 1 (Q)

Input		     Output

Q	S	R		Q

1	0	0		1

- Timing Diagram

- Initially (at time = 0), R, S and Q are at low voltage and Q is at high.

- At time = 1, R goes to high voltage. Q immediately goes to low voltage and Q goes to high.  S stays at low voltage.

- At time = 2, R goes to low voltage. Q stays at low voltage and Q stays

- at high, S stays at low voltage.

- At time = 3, S goes to high voltage. Q immediately goes to low voltage and Q goes to high.  R stays at low voltage.

- At time = 4, S goes to low voltage. Q stays at low voltage and Q stays at high. R stays at low voltage. After this, the RS flip-flop stays the way it was initially.

- Characteristic Table

Qt	S	R	Qt+1

0	0	0	 0

0	0	1	 0

0	1	0	 1

0	1	1 	 -

1	0	0	 1

1	0	1	 0

1	1	0	 1

1	1	1 	 -

- NAND Implementation

- Active-low inputs

- Characteristic Table

Qt	S	R	Qt+1

0	0	0	 -

0	0	1	 1

0	1	0	 0

0	1	1	 0

1	0	0	 -

1	0	1	 1

1	1	0	 0

1	1	1	 1

10.3	SR Flip-Flop

10.3.1	Clocked RS (Reset-Set) Flip-Flop

- Description:

- Known as the Set-Reset Flip-Flop as well.

- Two inputs to manipulate the state of the flip-flop.

- Excitation Table:

S	R	Qt+1 	(State)

0	0	  Qt 	(Hold)

0	1	  0 	(Reset)

1	0	  1 	(Set)

1	1	  - 	(Undefined)

- Characteristic Table

Qt	S	R	Qt+1

0	0	0	 0

0	0	1	 0

0	1	0	 1

0	1	1	 -

1	0	0	 1

1	0	1	 0

1	1	0	 1

1	1	1	 -

10.3.2	Level-triggered SR Flip-Flop

- Block Diagram

- Logic Diagram

10.3.3	Edge-triggered SR Flip-Flop

- Block Diagram

- Logic Diagram

10.4	D Flip-Flop

- Description:

- Known as the Delay Flip-Flop.

- Only one input, that always sets the output accordingly.

- Excitation Table:

D	Qt+1 (State)

0	    0

1	    1

- Block Diagram

- Characteristic Table

Qt	D	Qt+1

0	0	 0

0	1	 1

1	0	 0

1	1	 1

- Logic Diagram

10.5	T Flip-Flop

- Description:

- Known as the Toggle Flip-Flop.

- One input to manipulate the state of the flip-flop.

- Excitation Table:

T	Qt+1	(State)

0	  Qt 	(Hold)

1	  Qt 	(Toggle)

- Block Diagram

- Characteristic Table

Qt	T	Qt+1

0	0	 0

0	1	 1

1	0	 1

1	1	 0

- Logic Diagram

10.6	JK Flip-Flop

- Description:

- Two inputs.

- Excitation Table:

J	K	Qt+1 	(State)

0	0	  Qt 	(Hold)

0	1	  0 	(Reset)

1	0	  1 	(Set)

1	1	  Qt	(Toggle)

- Block Diagram

- Characteristic Table

Qt	J	K	Qt+1

0	0	0	 0

0	0	1	 0

0	1	0	 1

0	1	1	 1

1	0	0	 1

1	0	1	 0

1	1	0	 1

1	1	1	 0

- Logic Diagram

10.6.1	Summary of Flip-Flop Characteristics

- RS Flip-Flops:

- Cannot have both inputs active.

- Limited applications.

- D  Flip-Flops:

- Single input.

- Output follows input.

- JK Flip-Flops:

- No limitation on inputs.

- If both inputs active-toggles.

- T Flip-Flops:

- Single input.

- Output toggles when input is active.

10.7	Exercises

- List the binary outputs at Q and Q for the following flip-flops:

- a.

| 0 S Q ? ### CLK ## R Q ? FF | 1 | 0 |  | 1 |  |  | 0 |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f |  | e |  | d |  | c |  | b |  | a |  |
| 0 |  | 1 |  |  |  | 0 |  |  | 1 |  | 0 |  |
| Q Q |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |

- b.

| 0 S Q ? ### CLK ## R Q ? FF |  |  | 1 |  |  |  | 1 |  | 0 |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f |  | e |  | d |  | c |  | b |  | a |  |  |
| 1 |  |  | 0 |  | 1 |  | 0 |  | 1 |  | 0 |  |  |
| Q Q |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |

- c.

| J Q ? ### CLK ## K Q ? FF | 1 |  | 0 |  |  | 1 |  |  | 0 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f |  | e |  | d |  | c |  | b |  | a |  |  |
|  | 1 |  | 0 |  |  |  | 1 |  |  |  | 0 |  |  |
| Q Q |  |  |  |  |  |  |  |  |  |  |  | 0 |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |

- d.

| J Q ? ### CLK ## K Q ? FF |  | 1 |  |  |  |  | 0 |  |  |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f |  | e |  | d |  | c |  | b |  | a |  |
|  | 1 |  | 0 |  | 1 |  |  |  | 0 |  |  |  |
| Q Q |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |

- e.

| 0 D Q ? ### CLK ## Q ? FF |  | 1 |  |  | 0 |  |  | 0 |  | 0 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f |  | e |  | d |  | c |  | b |  | a |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 0 |
| Q Q |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |

- e.

| 0 T Q ? ### CLK ## Q ? FF |  | 1 |  |  | 0 |  |  | 1 |  | 0 |  | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | f |  | e |  | d |  | c |  | b |  | a |  |
| Q Q |  |  |  |  |  |  |  |  |  |  |  | 0 |
|  |  |  |  |  |  |  |  |  |  |  |  | 1 |