CHAPTER 5 : BOOLEAN ALGEBRA 2

Chapter Objectives

At the end of this chapter, you would have learnt :

- simplification of boolean expressions using :

- Boolean algebra, and

- Karnaugh map method.

Topics

- Boolean laws

- Karnaugh maps

5.1	Laws of Boolean Algebra

A.A	= 	0	(Complementation)

A.1	= 	A

(Dominance Law)

A.0 	=	0

A.A 	= 	A	(Idempotent Law)

A + 1 	= 	1

(Dominance Law)

A + 0 	= 	A

A + A 	= 	A	(Idempotent Law)

A + A 	= 	1	(Complementation)

A 	= 	A	(Involution Law)

- Commutative Law

- AB 	= 	BA

- A + B 	= 	B + A

- Associative Law

- A(BC) 	= 	(AB)C 		= 	ABC

- A + (B + C) 	= 	(A+ B) + C 	= 	A + B + C

- Distributive Law

- A(B + C) 	= 	AB + AC

- A + (BC) 	= 	(A + B)(A + C)

- De Morgan’s Theorem

- ABC 	=	 A + B + C

- A + B + C 	=	 A B C

- Absorption Law

- A + AB 	=	A

- A(A + B) 	= 	A

- Expansion Law

- A 	= 	A(B + B)

- A 	= 	(A + B).(A + B)

- Other Useful Identities

A + A.B 	=	A + B

Redundancy Law

- A.(A + B) 	=	A.B

- (A + B).(B + C).(A + C)  =  A.B + B.C + C.A

5.1.1	Simplification using Boolean Laws

Example:

F 	=	 ABC + AC + BC

=	 ABC + [AC(B + B)] + [BC(A + A)]	{Expansion Law}

=	 ABC+[AB C + AB C] + [ABC+ABC]

=	 ABC + AB C + ABC + ABC

{ABC + ABC = ABC}

=	 AB(C + C) + AC(B + B)

{Distributive Law}

=	 AB + AC

{X + X = 1}

Example:

F	= 	(A + C)(A + B)

=	 AA + AB + AC + BC

=	 AB + AC + BC

{since AA = 0}

=	 AB + AC + BC(A + A)

{Expansion Law}

=	 AB + AC + ABC + ABC

=	 AB(1 + C) + AC(1 + B)

=	 AB + AC

- For Boolean Algebra simplification, it is difficult to tell what is the most simplified form to a Boolean function

- Based on experience.

- Can double-check using K-map method.

Exercises using Boolean Laws

- AB + A + AB

- AB + ABC + A (B + AB)

- AB + AC + ABC ( AB + C )

5.2	Karnaugh Map

- For n variables, the K-map consists of 2n cells.

- Each cell represents a unique combination of the variables.

5.2.1	K-Map Structure

- 2-variable map

A

B

|  | 0 | 1 |
| --- | --- | --- |
| 0 |  |  |
| 1 |  | 3 |

- Example for the cell marked 3, AB = 112

- 3-variable map

A

B

|  | 0 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 |  |  | 6 |  |
| 1 |  |  |  |  |

- Example for the cell marked 6, ABC = 1102

- 4-variable map

A

B

|  | 0 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  |  |  |
| 01 |  |  | 13 |  |
| 11 |  |  |  |  |
| 10 |  |  |  |  |

- Example for the cell marked 13, ABCD =11012

5.2.2	Plotting the K-Maps

From  boolean expression:	f(A,B,C,D) = ACD + AB + D

- D 	:	Fill all the cells with D with 1.

- AB	: 	Fill all the cells with AB with 1.

- ACD	: 	Fill all the cells with A and CD with 1.

- Fill the rest of the cells with 0.

- Four variable map:

AB

CD

|  | 0 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  |  |  |
| 01 |  |  |  |  |
| 11 |  |  |  |  |
| 10 |  |  |  |  |

From the truth table:

| m | A | B | C | D | F |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 0 |
| 2 | 0 | 0 | 1 | 0 | 1 |
| 3 | 0 | 0 | 1 | 1 | 1 |
| 4 | 0 | 1 | 0 | 0 | 1 |
| 5 | 0 | 1 | 0 | 1 | 0 |
| 6 | 0 | 1 | 1 | 0 | 1 |
| 7 | 0 | 1 | 1 | 1 | 0 |
| 8 | 1 | 0 | 0 | 0 | 0 |
| 9 | 1 | 0 | 0 | 1 | 0 |
| 10 | 1 | 0 | 1 | 0 | 1 |
| 11 | 1 | 0 | 1 | 1 | 0 |
| 12 | 1 | 1 | 0 | 0 | 1 |
| 13 | 1 | 1 | 0 | 1 | 1 |
| 14 | 1 | 1 | 1 | 0 | 0 |
| 15 | 1 | 1 | 1 | 1 | 0 |

- Four variable map:

AB

CD

|  | 0 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 |  |  |  |  |
| 01 |  |  |  |  |
| 11 |  |  |  |  |
| 10 |  |  |  |  |

5.2.3	Simplification by K-Maps

- Steps:

- Plotting K-Map:

- Either boolean expression or truth table.

- Grouping of cells.

- Forming the equation.

5.2.4	Rules for Grouping

- All cells with 1 should be grouped at least once.

- Groups should be as large as possible.

- Each group should have 2n cells.

- Grouped cells can be reused if they help make the next group larger.

5.2.5	Simplification using K-Map

Do not loop these two ‘1’s together because they are already included in another group.

- Simplify:

AB

CD

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 1 | 1 | 1 | 0 |
| 01 | 0 | 0 | 1 | 0 |
| 11 | 1 | 0 | 0 | 0 |
| 10 | 1 | 1 | 0 | 1 |

F 	=	 A D + A B C + BCD + A BC

- Simplify:

AB

CD

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 0 | 0 |
| 01 | 1 | 0 | 0 | 1 |
| 11 | 1 | 0 | 0 | 1 |
| 10 | 0 | 0 | 1 | 0 |

F 	=

Don’t care condition

- Cell can be ‘0’ or ‘1’ (indicated by ‘X’).

- Used in grouping if it helps the simplification.

- Not necessary to use every ‘X’.

- Any group should not contain only ‘X’.

Using don’t care cells

Example:

AB

CD

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 00 | x | x |  | 1 |
| 01 |  |  |  | 1 |
| 11 |  |  |  |  |
| 10 | 1 | 1 | x | 1 |

This group is unnecessary

Using ‘don’t care’

f(A,B,C,D) 	=

5.2.6	Derivation of NAND Implementation

- Simplify K-map in SOP form.

- Apply double inversion to expression.

- Apply De Morgan’s theorem.

Example:

F	=	 AB + CD + E

F	=	 (AB + CD + E)

F	=	 (AB)(CD)(E)

5.2.7	Derivation of NAND-NAND Implementation

- AND-OR implementation

OR

2-Level NAND-NAND implementation

5.2.8	Derivation of NOR Implementation

- Simplify K-MAP in POS form.

- Apply double inversion to expression.

- Apply De Morgan’s theorem.

Example:

F	= 	(A + B)(A + C)(B + C)

F	=	(A + B)(A + C)(B + C)

F	= 	(A + B) + (A + C) + (B + C)

5.2.9	Derivation of NOR-NOR Implementation

- OR-AND implementation

OR

2-level NOR-NOR implementation

5.2.10	Design in Boolean Logic

- Steps:

- Develop the truth-table.

- Write SOP/POS forms of the function.

- Simplify the Boolean expression.

- If implemented in NAND-NAND/NOR-NOR form, use De Morgan’s theorem on Boolean expression derived.

- Design using logic gates.

Example:

Design a gating network with three inputs A, B and C, that is to have a “1" output only when two or three of the inputs have a 1 value. Implement this function in NAND-NAND form.

Example:

Design a two-level combinational logic circuit which will produce a “1" only when the number of 1’s in a set of three input variables A, B and C is even (i.e. A parity checker)

5.3	Exercises

- Boolean Algebra Simplification:

- F1  =  A B C D + A B C D + A B C D

- F2  =  A B C + A B C+ A B C + A B C

- F3  =  A B C + A B C + A B C + A B C + A B C

- Using K-map: Simplify

a.	b.

AB		AB

CD	CD

|  | 1 | 0 | 0 | 0 |  | 1 | 1 | 0 | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 1 | 1 | 0 |  | 1 | 0 | 0 | 1 |

c.	d.

AB		AB

CD	CD

|  | 0 | 0 | 1 | 1 |  | 1 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 |  | 1 | 1 | 0 | 0 |
|  | 1 | 0 | 0 | 1 |  | 0 | 1 | 1 | 0 |
|  | 0 | 0 | 1 | 1 |  | 0 | 0 | 0 | 0 |

e.	(Using ‘Don’t Care’)

AB

CD

|  | 0 | 0 | 1 | 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 |  |  |  |  |  |
|  | 1 | 0 | 0 | 1 |  |  |  |  |  |
|  | 0 | 0 | 1 | 1 |  |  |  |  |  |

The truth table illustrates a seven-segments LCD display design, in accordance to the following diagrams.

| A3 | A2 | A1 | A0 | a | b | c | d | e | f | g |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 1 |
| 0 | 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 0 | 0 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 |
| 0 | 0 | 1 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 |
| 0 | 1 | 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 |
| 0 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |
| 0 | 1 | 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 0 | x | x | x | x | x | x | x |
| 1 | 0 | 1 | 1 | x | x | x | x | x | x | x |
| 1 | 1 | 0 | 0 | x | x | x | x | x | x | x |
| 1 | 1 | 0 | 1 | x | x | x | x | x | x | x |
| 1 | 1 | 1 | 0 | x | x | x | x | x | x | x |
| 1 | 1 | 1 | 1 | x | x | x | x | x | x | x |

- From the above truth table, draw the Karnaugh-map for segments a and b only, and derive the minimized terms for these segments.

- Implement the circuit of a-segment using NAND gates, and b-segment using only NOR gates.