CHAPTER 9 : COMBINATIONAL CIRCUIT 2

Chapter Objectives

At the end of this chapter you would have learnt:

- how to design multiplexors, demultiplexors, decoders, encoders and their applications.

Topics

- Multiplexor

- Demultiplexor

- Decoder

- Encoder

9.1	Multiplexor

- Description:

- 2n data with n-bit select inputs, e.g. 8 data with 3 select.

- One output from one of the 2n data.

- Select lines (S) acts as a switch to determine which data is allowed to pass through.

9.1.1	2-to-1 MUX

If S is 0 then A0 will be allowed to pass through to B.

If S is 1 then A1 will be allowed to pass through to B.

- Block Diagram (2-to-1 MUX)

- Truth Table (2-to-1 MUX)

A1	A0	S	B

0	0	0	0

0	0	1	0

0	1	0	1

0	1	1	0

1	0	0	0

1	0	1	1

1	1	0	1

1	1	1	1

Therefore, B = A0S + A1S

A1A0

S

|  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 | 1 |

- Logic Diagram (2-to-1 MUX)

9.1.2	2-to-1 MUX as a Universal Gate

- As an OR gate

- Based on the truth table (m4, m5, m6, m7), these four terms are such that A1 is 1; A0 & S in all combinations, the output B is that of an OR gate, hence,

- Block Diagram

- Truth Table

A1	A0	S	B = X + Y

1	0	0	   0

1	0	1	   1

1	1	0	   1

1	1	1	   1

- As an AND gate

- Based on the truth table (m0, m1, m4, m5), these four terms are such that A0 is 0; A1 and S in all combinations, the output B is that of an AND gate, hence,

- Block Diagram

- Truth Table

A0	A1	S	B = X.Y

0	0	0	   0

0	0	1	   0

0	1	0	   0

0	1	1	   1

- As a NOT gate

- Based on the truth table (m2, m3), these two terms are such that A1 is 0; A0 is 1; S in all combinations, the output B is that of an NOT gate, hence,

- Block Diagram

- Truth Table

A1	A0	S	B = X

0	1	0	   1

0	1	1	   0

9.2	Demultiplexor

- Description:

- One data with n-bit select inputs.

- 2n data outputs, e.g. 3 select for 8 data.

- Select lines act as a switch to determine which output data line to pass through.

9.2.1	1-to-2 DEMUX

- Block Diagram (1-to-2 DEMUX)

- Truth Table (1-to-2 DEMUX)

A	S	B1	B0

0	0	0	0

0	1	0	0

1	0	0	1

1	1	1	0

A		A

S		S

|  | 0 | 1 |  | 0 | 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 | 0 |

B1 = AS	B0 = AS

- Logic Diagram (1-to-2 DEMUX)

Example:

- Multiplexing and Demultiplexing of registers into a parallel adder.

- The objective is to save the number of parallel adder and bus connection among the registers.

- Illustration

- When S1S0 = 00, MUX1 will select register A;

- When D1D0 = 10, MUX2 will select register B.

- Both registers A and B content will be added and passed to the DEMUX.

- When X1X0 = 11, the result will be passed to register C for storage.

Example:

Construct a 16 x 1 MUX with two 8 x 1 and one 2 x 1 MUX.  Use block diagrams for the three multiplexers.

9.3	 Decoder

- Description:

- n bit binary inputs.

- 2n outputs.

- Select from n--bit inputs an unique output of 2n lines, e.g. 3--to--8 decoding.

- Block Diagram (2-to-4 decoder)

- Truth Table (2-to-4 decoder)

A1	A0	B3	B2	B1	B0

0	0	0	0	0	1

0	1	0	0	1	0

1	0	0	1	0	0

1	1	1	0	0	0

A1		A1

A0		A0

|  | 0 | 1 |  | 0 | 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 1 | 0 |

B0 = A1A0	B1 = A1A0

A1		A1

A0		A0

|  | 0 | 1 |  | 0 | 1 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 0 | 0 | 0 |
| 1 | 0 | 0 | 1 | 0 | 1 |

B2 = A1A0	B3 = A1A0

- Logic Diagram (2-to-4 Decoder)

Example:

- 16 cells memory access matrix.

- Notes: 	SR 	 	select row

- SC 		select column

- I  		Input

- O 		Output

- A1A0 is used to select the rows

- A3A2 is used to select the columns

- Illustration

- When A1A0 = 00, the first row of the matrix will be selected.

- When A3A2 = 00, the first column will be selected.

- The intersection cell will be the one activated.

Example:

Implement a full-adder circuit with a decoder and two OR gates.

Step 1:	Provide a truth table for a full-adder.

x	y	z	C	S

0	0	0	0	0

0	0	1	0	1

0	1	0	0	1

0	1	1	1	0

1	0	0	0	1

1	0	1	1	0

1	1	0	1	0

1	1	1	1	1

Step 2:	Obtain the functions for this combinational circuit in sum of minterms:

S(x, y, z) =  (1, 2, 4, 7)

C(x, y, z) =  (3, 5, 6, 7)

9.4	 Encoder

- Description:

- 2n inputs (or fewer) such that only one is active at any time

- n bit binary outputs

- Based on one active line out of 2n, the binary equivalent of n-bit is determined, e.g. 8-to-3 encoder

- Truth Table of Octal-to-Binary Encoder

Inputs					   Outputs

D0	D1	D2	D3	D4	D5	D6	D7	x	y	z

1	0	0	0	0	0	0	0	0	0	0

0	1	0	1	0	0	0	0	0	0	1

0	0	1	0	0	1	0	0	0	1	0

0	0	0	1	0	1	0	0	0	1	1

0	1	0	0	1	0	0	0	1	0	0

0	1	0	1	0	1	0	0	1	0	1

0	1	1	0	1	0	1	0	1	1	0

0	1	1	1	1	0	0	1	1	1	1

Octal-to-binary encoder

9.4.1	Priority Encoder

- Description:

- Includes the priority function.

- If two or more inputs are equal to 1 at the same time, the input having the highest priority will take precedence.

- Block Diagram (4-to-2 priority encoder)

- Truth Table (4-to-2 priority encoder)

A3	A2	A1	A0	B1	B0

0	0	0	0	0	0

0	0	0	1	0	0

0	0	1	0	0	1

0	0	1	1	0	1

0	1	0	0	1	0

0	1	0	1	1	0

0	1	1	0	1	0

0	1	1	1	1	0

1	0	0	0	1	1

1	0	0	1	1	1

1	0	1	0	1	1

1	0	1	1	1	1

1	1	0	0	1	1

1	1	0	1	1	1

1	1	1	0	1	1

1	1	1	1	1	1

- Illustration

When A3A2A1A0 = 0111 (at row 8), input A2 has the highest priority; so regardless of the values of the other inputs, when this input is 1, the output for B1B0 is 10 (binary 2).

A3A2		A3A2

A1A0	A1A0

|  | 00 | 01 | 11 | 10 |  | 00 | 01 | 11 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 00 | 0 | 0 | 1 | 0 | 00 | 0 | 0 | 1 | 1 |
| 01 | 0 | 1 | 1 | 1 | 01 | 0 | 0 | 1 | 1 |
| 11 | 0 | 1 | 1 | 1 | 11 | 1 | 0 | 1 | 1 |
| 10 | 0 | 1 | 1 | 1 | 10 | 1 | 0 | 1 | 1 |

B1 = A3 + A2 	B0 = A3 + A2A1

- Logic Diagram (4-to-2 Priority Encoder)

- Example:

- 12 keys phone pad-encoding matrix	.

| Key | Binary |  | Key | Binary |  | Key | Binary |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 0001 |  | 2 | 0010 |  | 3 | 0011 |
| 4 | 0100 |  | 5 | 0101 |  | 6 | 0110 |
| 7 | 0111 |  | 8 | 1000 |  | 9 | 1001 |
| * | 1011 |  | 0 | 1010 |  | # | 1100 |

9.5	Exercises

- Design a 1-to-4 DEMUX, showing the truth table and logic diagram.

- Design a 4-to-1 MUX using two 2-to-1 MUXs.

- (You may use any other gates for connection)

- Design a 3 bit gray code decoder,

| Gray Code | Active Output |
| --- | --- |
| 000 | B0 |
| 001 | B1 |
| 011 | B2 |
| 010 | B3 |
| 110 | B4 |
| 111 | B5 |
| 101 | B6 |
| 100 | B7 |

by showing the truth table and the logic diagram.