CHAPTER 7 : TUTORIAL

- Express the following number in floating point format using 10 bit mantissa in sign modulus and 6 bit exponent in sign-modulus.

+0.777778

- The ‘ADD 3’ block takes a 3-bit input and gives a 3-bit output. The values at the output are greater than the input by 3. If the output value is too big to be represented then all the output bits are set to 1.

- Construct the truth-table for the above block.

- Derive the K-map from the truth table.

- Draw the circuit diagram for the above block using NAND gates only.

- By assuming the use of a 16-bit word with 10 bits for the mantissa (normalised) and 6-bits for the exponent and both in 2’s complement format, perform the addition using floating point arithmetic. Show each step of your calculation.

- O.199116 + O.DAD16

- O.FOOD16 - BAD.FOOD16

- 19A3.516 - FC816

- An encoder is a circuit that generates binary code corresponding to the digital input value. Given below is the truth table for a 2 to 1 encoder. Note that the input to the encoder must have just one bit which is non zero.

D1	D0	A0

0	1	0

1	0	1

With the above reference, do the following:

- Produce the truth table;

- Produce the SOP and POS expression for the output bits;

- Simplify A0, A1 using a K-maps;

- Draw the circuit using NAND gates only.